RECIVERS DEWAR CONTROLS

REMOTE CONTROLS

COMMENTS

| S.Ware Refer. | H.Ware Refer. | Signals Lable Name | Electrical Functions | Ext I/O Bits Value | SWare I/O Bits Value |  | MF22GHz | 7GHz Tolerance (Location) | L-P |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

OL2

| I 19 | IN 3 | REMOTE/LOCAL | Light = Remote Unlight = Local | 0 1 | 1 0 |  | NO | NO | NO |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| O 0 | OUT 0 | SYNT1_SEL/SYNT2_SEL | Opened = SYNT1 Mode Selected Closed = SYNT2 Mode Selected | 1 0 | 0 1 |  | NO | NO | NO |  |  |
| O 1 | OUT 1 | SPARE |  |  |  |  | NO | NO | NO |  |  |
| O 2 | OUT 2 | SPARE |  |  |  |  | NO | NO | NO |  |  |
| O 15 | OUT 15 | SPARE |  |  |  |  | NO | NO | NO |  |  |
| I 16 | IN 0 | SYNT1_STATUS | Light = Synt1 Mode Selected Unlight = Synt1 Mode Unselected | 0 1 | 1 0 |  | NO | NO | NO |  |  |
| I 17 | IN 1 | SYNT2_STATUS | Light = Synt2 Mode Selected Unlight = Synt2 ModeUnselected | 0 1 | 1 0 |  | NO | NO | NO |  |  |
| I 18 | IN 2 | OL2_UNLOCK | Light = OL2 Unlocked Unlight = OL2 Locked | 0 1 | 1 0 |  |  | NO | NO |  |  |
| I 31 | IN 15 | SPARE |  |  |  |  | NO | NO | NO |  |  |

DEWAR CRYO TEMPERATURE  SENSORS

| AD 8 | AD 8 | TEMP_CRYO_1 | T [K] (Firmware Converted) |  |  |  | ?? 20K ?? +100% (LNA L1) | 65K +40% (Dito) | ? |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| AD 9 | AD 9 | TEMP_CRYO_2 | T [K] (Firmware Converted) |  |  |  | ?? 70K ?? +30% (Piastra) | 100K +30% (Finestra) | ? |  |  |
| AD 11 | AD 11 | TEMP_CRYO_3 | T [K] (Firmware Converted) |  |  |  | ?? 20K ?? +80% (Forca) | 20K +100% (LNA) | ? |  |  |
| AD 12 | AD 12 | TEMP_CRYO_4 | T [K] (Firmware Converted) |  |  |  | ?? 70K ?? +40% (Dito) | 35K +80% (Finestra) | ? |  |  |

VACUUM SENSOR

| O 4 | OUT 4 | VCM_SENSOR_ON/OFF | Opened = Sensor OFF Closed = Sensor ON | 1 0 | 0 1 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| AD 10 | AD 10 | VCM_SENSOR | P [mB] (Firmware Converted) |  |  |  | 5x10-7 mB +20000% | 5x10-7 mB +20000% | 5x10-7 mB +20000% |  |  |

POWER BOX

| I 26 | IN 10 | REMOTE/LOCAL | Light = Remote Unlight = Local | 0 1 | 1 0 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| O 7 | OUT 7 | VCMVALVE_EN (priority) | Opened = Valve modality DISABLED Closed = Valve OPENS AT START PUMP To be setted before Pump ON if dewar is NOT EMPTY | 1 0 | 0 1 |  |  |  |  |  |  |
| O 10 | OUT 10 | VCMVALVE_EN_DLY (no priority) | Opened = Valve modality DISABLED Closed = Valve OPENS AT PUMP REGIME To be setted before Pump ON if dewar is already EMPTY | 1 0 | 0 1 |  |  |  |  |  |  |
| O 5 | OUT 5 | VCMPUMP_ON/OFF | Opened = Pump OFF Closed = Pump ON | 1 0 | 0 1 |  |  |  |  |  |  |
| O 6 | OUT 6 | VCMPUMP_SFTSTR | Opened = Pump Soft Start DISABLED Closed = Pump Soft Start ENABLED NOT USED NOW (ONLY FOR VARIAN PUMP) | 1 0 | 0 1 |  | NO | NO | NO |  |  |
| I 21 | IN 5 | VCMPUMP_STATUS | Light = Pump ON and NORMAL Unlight = Pump OFF or RAMP | 0 1 | 1 0 |  |  |  |  |  |  |
| I 22 | IN 6 | VCMPUMP_FAULT (optional status) | Light = Pump FAULT Unlight = Pump OK | 0 1 | 1 0 |  |  |  |  |  |  |
| I 23 | IN 7 | VCMVALVE_STATUS | Light (AC) = Valve OPEN Unlight (AC) = Valve CLOSED | 0 1 | 1 0 |  |  |  |  |  |  |
| O 8 | OUT 8 | CLHEAD_ON/OFF | Opened = Head OFF Closed = Head ON | 1 0 | 0 1 |  |  |  |  |  |  |
| I 24 | IN 8 | CLHEAD_STATUS | Light (AC) = Head ON Unlight (AC) = Head OFF | 0 1 | 1 0 |  |  |  |  |  |  |
| O 9 | OUT 9 | RESIST_START | Opened = OFF Transition Opened-Closed = Start Heating Cycle (Timed) NOT USED NOW | 1 1=>0=>1 | 0 0=>1=>0 |  | NO | NO | NO |  |  |
| I 25 | IN 9 | RESIST_STATUS | Light (AC) = Heating Cycle (Timed) Unlight (AC) = OFF NOT USED NOW | 0 1 | 1 0 |  | NO | NO | NO |  |  |

MARK NOISE GENERATOR

| O 11 | OUT 11 | MGEN_ON/OFF | Opened = Mark OFF Closed = Mark ON | 1 0 | 0 1 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| O 12 | OUT 12 | MGEN_EXT_EN | Opened = External Synchronous Command Disabled Closed = External Synchronous Command Enabled | 1 0 | 0 1 |  |  |  |  |  |  |

SINGLE DISH / VLBI MODE

| O 13 | OUT 13 | SD_SEL (/VLBI_SEL) | Opened = Single Dish Mode Closed = VLBI Mode | 1 0 | 0 1 |  |  | NO | NO |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| O 14 | OUT 14 | VLBI_SEL (/SD_SEL) | Opened = VLBI Mode Closed = Single Dish Mode | 1 0 | 0 1 |  |  | NO | NO |  |  |
| I 29 | IN 13 | SD_SEL_STATUS | Light = Single Dish Mode Unlight = No Single Dish Mode | 0 1 | 1 0 |  |  | NO | NO |  |  |
| I 30 | IN 14 | VLBI_SEL_STATUS | Light = VLBI Mode Unlight = No VLBI Mode | 0 1 | 1 0 |  |  | NO | NO |  |  |

AUX TEMPERATURE SENSOR

| AD 13 | AD 13 | TEMP 1 | T [C] (Firmware Converted) |  |  |  | 45°C +20/-70% (OL2) | NO | NO |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| AD 14 | AD 14 | TEMP 2 | T [C] (Firmware Converted) |  |  |  | 25°C +20% (Envirom.) | 25°C (Envirom.) | ??45°C (Envirom.) |  |  |
| AD 15 | AD 15 | TEMP 3 | T [C] (Firmware Converted) |  |  |  | NO | NO | NO |  |  |

LOCAL CONTROLS (THEY ARE THE SAME FOR ALL RECEIVERS)

POWER BOARD PANEL

| SW 1 | VP Vacuum Pump command Switch 1-0-2 | Command: 1 = Pump ON Soft Start (ONLY FOR VARIAN PUMP) 0 = Pump OFF 2 = Pump ON Status: ON Green LED Lights = Pump On NRM Yellow LED Lights = Pump Regime FAIL Red LED Lights = Pump Fail |
| --- | --- | --- |
| SW 2 | VV Vacuum Valve command Switch 1-0-2 | Command: 1 = Valve Opens at Sart Pump 0 = Valve Always Closed 2 = Valve Opens at Pump Regime Status OPEN Green LED Lights = Vacuum Valve Opened VENT Green LED Lights = Vent Valve Opened (timed) |
| SW 3 | CH Cold Head command Switch 1-2 | Command: 1 = OFF 2 = ON Status: ON Green Lights = Cold Head On |
| SW 4 | HR Heat Resistors command (Cycle Enable) Switch 1-2 | Command: 1 = OFF 2 = Enable Starting Cycles Status: EN Green LED Lights = Starting Cycles Enable HEAT Green LED Light = Resistors Heating |
| SW 5 | HR Heat Resistors command (Cycle Starting) Pushbutton | Command: Released = OFF Push and Release = Start Cycle (timed) |
| SW 6 | LOC REM LOCal REMote command Switch 1-2 | Command: 1 = Local Commands Enabled 2 = Remote Commands Enabled Status: LOC Green LED Lights = Remote REM Green LED Lights = Remote |